Code Documentation

- Overview

This MATLAB script is designed to visualize in real-time the behavior of the solar tracking system implemented using an Arduino-based dual-mode controller (PID & non-PID). The goal of this script is to monitor key performance indicators such as:

- Light intensity readings from LDR sensors (LDR1 & LDR2)

- Servo motor angle (representing panel position)

- Current mode of operation (Automatic vs Manual)

- Connection to Arduino System

The MATLAB script receives data via the serial port from the Arduino microcontroller.

The Arduino code sends the data through Serial.print() every 20 milliseconds in a comma-separated format, as follows:

<time_ms>,<ldr1>,<ldr2>,<angle>,<pid_output>,<mode>

Example line from serial output:

12560,43,59,22.5,1.84,1

Note:

This MATLAB script will not function unless the Arduino system is connected via USB, and the correct COM port and baud rate (115200) are set in the script:

port = "COM6";      % Change this to match your system

baud = 115200;

If no Arduino is connected, the script will throw a connection error or wait indefinitely for data.

For best results, make sure:

- The Arduino code is already uploaded and running.

- The Serial Monitor on the Arduino IDE is closed (to avoid port conflict).

- The hardware is powered and functional.

- What the Code Does

1. Initial Setup

- Connects to the specified COM port.

- Initializes data buffers and a live figure window with 3 subplots:

- LDR Readings (0–100%)

- Panel Angle (−90° to +90°)

- Operating Mode (Auto / Manual)

2. Live Data Loop

- Continuously reads and parses the serial data stream.

- Buffers the last 1000 points for plotting.

- Updates the plots every 100 ms.

3. Data Filtering

- Automatically skips any malformed or incomplete lines.

- Uses circular buffering to keep memory usage efficient.

- Advantages of This Visualization

- Allows real-time observation of how the system responds to changing light conditions.

- Helps compare the performance between PID-controlled and non-PID-controlled behavior.

- Provides visual evidence of oscillations, stabilization time, and accuracy.

- Use Case

This script is primarily used to evaluate and present the real-time response of the solar tracking system during:

- Lab testing and debugging

- Performance comparison

- Academic present

- Appendix

Appendix A: the Matlab code (live script):

%% Solar Tracker Live Visualization (Improved & Stable Version)

clear; clc; close all;

%% Serial Setup

port = "COM6";       % <-- Replace with your actual COM port

baud = 115200;

try

s = serialport(port, baud);

configureTerminator(s, "LF");

flush(s);

pause(2);  % Let Arduino initialize

disp(['Connected to ' port]);

catch ME

error('Connection failed: %s', ME.message);

end

%% Discard initial unstable readings

for i = 1:10

if s.NumBytesAvailable > 0

readline(s);

end

pause(0.05);

end

%% Figure Setup

figure('Color','w','Position',[100 100 900 600]);

sgtitle('Solar Tracker System Response','FontSize',14,'FontWeight','bold');

% Subplot 1: LDR Readings

ax1 = subplot(3,1,1);

hLDR1 = plot(nan, nan, 'r-', 'LineWidth',1.5, 'DisplayName','LDR1'); hold on;

hLDR2 = plot(nan, nan, 'b-', 'LineWidth',1.5, 'DisplayName','LDR2');

title('LDR Readings (0–100%)');

ylabel('Light Intensity (%)');

ylim([0 100]);

grid on;

legend('Location','best');

% Subplot 2: Panel Angle

ax2 = subplot(3,1,2);

hAngle = plot(nan, nan, 'Color',[0 0.5 0], 'LineWidth',1.5);

title('Panel Angle');

ylabel('Angle (°)');

ylim([-90 90]);

grid on;

yline(0,'--','0° Horizon');

% Subplot 3: Control Mode (Auto/Manual)

ax3 = subplot(3,1,3);

hMode = stairs(nan, nan, 'k-', 'LineWidth',1.5);

title('Control Mode');

ylabel('Mode');

xlabel('Time (s)');

yticks([0 1]);

yticklabels({'Manual','Auto'});

ylim([-0.1 1.1]);

grid on;

%% Data Buffers

bufferSize = 1000;

timeBuffer  = nan(1, bufferSize);

ldr1Buffer  = nan(1, bufferSize);

ldr2Buffer  = nan(1, bufferSize);

angleBuffer = nan(1, bufferSize);

modeBuffer  = nan(1, bufferSize);

ptr = 1;

%% Plot Refresh Setup

updateInterval = 0.1;  % seconds

lastUpdateTime = 0;

startTime = tic;

disp('Reading data... Close figure to stop.');

%% Main Loop

while ishandle(gcf)

while s.NumBytesAvailable > 0

try

line = readline(s);

% Check data structure (must have 5 commas → 6 fields)

if count(line, ',') ~= 5

continue;

end

data = sscanf(line, '%f,%f,%f,%f,%f,%f');

if numel(data) ~= 6

continue;

end

% Optional: skip idle responses (both LDRs zero)

if data(2) == 0 && data(3) == 0

continue;

end

% Store in circular buffer

timeBuffer(ptr)  = data(1) / 1000;  % Convert ms to seconds

ldr1Buffer(ptr)  = data(2);

ldr2Buffer(ptr)  = data(3);

angleBuffer(ptr) = data(4);

modeBuffer(ptr)  = data(6);  % skip PID value (data(5))

ptr = mod(ptr, bufferSize) + 1;

catch

% Skip malformed lines silently

end

end

% Refresh plots every updateInterval

if toc(startTime) - lastUpdateTime >= updateInterval

valid = ~isnan(timeBuffer);

t = timeBuffer(valid);

if isempty(t), continue; end

set(hLDR1, 'XData', t, 'YData', ldr1Buffer(valid));

set(hLDR2, 'XData', t, 'YData', ldr2Buffer(valid));

set(hAngle, 'XData', t, 'YData', angleBuffer(valid));

set(hMode,  'XData', t, 'YData', modeBuffer(valid));

% Auto X-axis scrolling (last 10 seconds)

xLimits = [max(0, t(end)-10), max(10, t(end)+1)];

set(ax1, 'XLim', xLimits);

set(ax2, 'XLim', xLimits);

set(ax3, 'XLim', xLimits);

drawnow limitrate;

lastUpdateTime = toc(startTime);

end

pause(0.001);  % Prevent CPU overload

end

%% Cleanup

clear s;

disp('Stopped.');

Appendix B: the Aurduino IDE code :

#include <Wire.h>

#include <U8x8lib.h>

#include <Servo.h>

#include <IRremote.h>

U8X8_SSD1306_128X64_NONAME_HW_I2C oled(/* reset=*/ U8X8_PIN_NONE);

Servo solarServo;

// === Pin Definitions ===

#define SERVO_PIN     4

#define LDR1_PIN      A0

#define LDR2_PIN      A2

#define BUTTON_PIN    2

#define GREEN_LED     6

#define YELLOW_LED    7

#define IR_PIN        3

#define BUZZER_PIN    5

#define IR_LEFT   0xF40B0707

#define IR_RIGHT  0xF8070707

const int PHYS_MIN = 25;

const int PHYS_MAX = 165;

const int PHYS_MID = 95;

const int DISP_MIN = -90;

const int DISP_MAX = 90;

//const float Kp = 1.2;

//const float Ki = 0.08;

//const float Kd = 0.5;

const float Kp = 10;

const float Ki = 0.08;

const float Kd = 0.5;

int currentPulse = PHYS_MID;

bool autoMode = true;

bool lastButtonState = HIGH;

unsigned long lastDebounceTime = 0;

const unsigned long debounceDelay = 200;

float error = 0, lastError = 0, integral = 0, derivative = 0;

unsigned long lastTime = 0;

void setup() {

Serial.begin(115200);

Wire.begin();

oled.begin();

oled.setFont(u8x8_font_chroma48medium8_r);

oled.clearDisplay();

oled.drawString(0, 0, " Solar Tracker");

pinMode(BUTTON_PIN, INPUT_PULLUP);

pinMode(GREEN_LED, OUTPUT);

pinMode(YELLOW_LED, OUTPUT);

pinMode(BUZZER_PIN, OUTPUT);

digitalWrite(BUZZER_PIN, LOW);

digitalWrite(GREEN_LED, HIGH);

digitalWrite(YELLOW_LED, LOW);

IrReceiver.begin(IR_PIN, ENABLE_LED_FEEDBACK);

solarServo.attach(SERVO_PIN);

solarServo.write(PHYS_MID);

playStartupBeep();

delay(1000);

}

void loop() {

static unsigned long lastSendTime = 0;

const unsigned long sendInterval = 20;

handleModeToggle();

digitalWrite(GREEN_LED, autoMode);

digitalWrite(YELLOW_LED, !autoMode);

int ldr1 = readLDRPercent(LDR1_PIN);

int ldr2 = readLDRPercent(LDR2_PIN);

float pidOutput = 0;

float currentAngle = pulseToAngle(currentPulse);

if (autoMode) {

unsigned long now = millis();

float dt = (now - lastTime) / 1000.0;

lastTime = now;

error = ldr1 - ldr2;

integral += error * dt;

integral = constrain(integral, -100, 100);

derivative = (error - lastError) / dt;

pidOutput = Kp * error + Ki * integral + Kd * derivative;

lastError = error;

if (abs(error) > 3) {

currentPulse += constrain(pidOutput, -2, 2);

currentPulse = constrain(currentPulse, PHYS_MIN, PHYS_MAX);

solarServo.write(currentPulse);

}

} else {

if (IrReceiver.decode()) {

uint32_t code = IrReceiver.decodedIRData.decodedRawData;

if (code == IR_LEFT) currentPulse -= 5;

else if (code == IR_RIGHT) currentPulse += 5;

currentPulse = constrain(currentPulse, PHYS_MIN, PHYS_MAX);

solarServo.write(currentPulse);

IrReceiver.resume();

}

}

updateOLED(ldr1, ldr2, currentAngle);

if (millis() - lastSendTime >= sendInterval) {

Serial.print(millis()); Serial.print(",");

Serial.print(ldr1); Serial.print(",");

Serial.print(ldr2); Serial.print(",");

Serial.print(currentAngle, 1); Serial.print(",");

Serial.print(pidOutput, 2); Serial.print(",");

Serial.println(autoMode ? '1' : '0');

lastSendTime = millis();

}

}

void handleModeToggle() {

bool reading = digitalRead(BUTTON_PIN);

if (reading == LOW && lastButtonState == HIGH &&

(millis() - lastDebounceTime > debounceDelay)) {

autoMode = !autoMode;

lastDebounceTime = millis();

playToggleBeep();

}

lastButtonState = reading;

}

void updateOLED(int ldr1, int ldr2, float angle) {

oled.clearLine(1); oled.setCursor(0, 1);

oled.print(" L:"); oled.print(ldr1); oled.print("  R:"); oled.print(ldr2);

oled.clearLine(2); oled.setCursor(0, 2);

oled.print(" Mode: "); oled.print(autoMode ? "Auto" : "Manual");

oled.clearLine(3); oled.setCursor(0, 3);

oled.print(" Angle: ");

if (angle >= 0) oled.print("+");

oled.print(angle, 0);

oled.print((char)176); // degree symbol

}

int readLDRPercent(int pin) {

int raw = analogRead(pin);

return constrain(map(raw, 50, 950, 0, 100), 0, 100);

}

float pulseToAngle(int pulse) {

return map(pulse, PHYS_MIN, PHYS_MAX, DISP_MAX, DISP_MIN);

}

void playStartupBeep() {

digitalWrite(BUZZER_PIN, HIGH); delay(150);

digitalWrite(BUZZER_PIN, LOW);  delay(100);

digitalWrite(BUZZER_PIN, HIGH); delay(200);

digitalWrite(BUZZER_PIN, LOW);

}

void playToggleBeep() {

digitalWrite(BUZZER_PIN, HIGH);

delay(100);

digitalWrite(BUZZER_PIN, LOW);

}