- Introduction:

Solar energy systems are more efficient when the panel surface is oriented directly toward the sun. A solar tracking system improves efficiency by continuously aligning the panel with the sun’s position. This project aims to design a small-scale dual-mode solar tracking system, combining sensor-based automatic tracking with manual control, while also allowing real-time analysis through MATLAB

- Overview:

This project proposes the development of a smart solar tracking system capable of operating in two modes:

- Automatic Mode: The system automatically detects the direction of sunlight using light-dependent resistors (LDRs) and adjusts the solar panel position using a servo motor.

- Manual Mode: The panel can be controlled manually using an infrared (IR) remote.

To monitor the system’s behavior, real-time data will be transmitted to MATLAB for live visualization and analysis. The system aims to combine responsive control with a simple interface and real-time feedback.

- Objectives:

- Develop a functional solar tracking system that operates in both automatic and manual modes.

- Implement a PID controller to optimize alignment with the sun in automatic mode.

- Provide manual control using an IR remote for flexibility.

- Integrate LCD, LEDs, and a buzzer for user-friendly interaction.

- Transmit system data (light readings, angle, mode) from Arduino to MATLAB.

- Create a MATLAB script for real-time visualization and response monitoring.

- System Scope:

The system will include the following core features:

- Sunlight tracking using two LDR sensors

- Servo motor control for adjusting panel angle

- Dual-mode switching via push button

- Manual control through IR remote

- LCD screen displaying system data (mode, angle, sensor readings)

- Data logging to MATLAB via serial communication

- MATLAB live plots showing panel angle, PID response, LDR values, and control mode.

- Required Equipment:

In this project the component that will be used is the following :

- Arduino Uno (or ESP32 if upgraded later)

- 2× LDR (Light Dependent Resistors)

- 1× Servo Motor (SG90 or equivalent)

- 1× IR Receiver (HW-511 or similar)

- 1× IR Remote

- 1× LCD 16×2 with I2C Module

- 2× LEDs (for mode indication)

- 1× Push Button (for mode switching)

- 1× Buzzer (for audible feedback)

- Basic solar panel (or simulation model)

- Breadboard and jumper wires

- MATLAB (for response visualization)

- Expected Deliverables:

A working prototype of the dual-mode solar tracker

Real-time communication between Arduino and MATLAB

Visual display of tracking behavior through MATLAB plots

A simple interface with LCD, LEDs, and buzzer for feedback

Clear demonstration of the system’s performance in both modes